- SL45/6688

- Level 2.5

Repair Documentation

- V 1.2

- Table of Contents:

1	Introduction	3

2	Bottom Connector (Lumberg)	4

3	Display Connector	8

4	Keyboard LEDs	12

5	Keyboard Connector	15

6	Irda Diode	18

7	Antenna Connector	21

8	Resistor R384	25

9	Voltage Protection Diode V200	29

# Introduction

SL45 product family, internally referred to as U35,  consists of 2 different dualband handsets (GSM-900 and GSM-1800), which can easily be distinguished from the second block of the partnumber printed on the IMEI label. There is a standard type SL45 and a Chinese type called 6688.

- Partnumber on IMEI label:

- SL45:		S30880-S4800-#1xx

- 6688:		S30880-S4800-#8xx		

,while # may be any letter (A-Z) and xx may be any number from 10,20,30 ....90.

- Important:

The SL45 and the 6688 use a different hardware (RF/control module). This is due to the fact that for the Chinese type there is a bigger flash memory size required for the Chinese characters.

The information in this manual applies to both types, SL45 and 6688, unless otherwise noted.

This manual is intended to help you carry out repairs on level 2.5, meaning limited component repairs. Failure highlights are documented and should be repaired in the local workshops.

It must be noted that all repairs have to be carried out in an environment set up according to the ESD (Electrostatic Discharge Sensitive Devices) regulations defined in international standards.

If you have any questions regarding the repair procedures or spare parts do not hesitate to contact our technical support team in Kamp-Lintfort, Germany:

- Tel.: +49 2842 95 4666

Fax: +49 2842 95 4302

e-mail: dominik.schnoor@klf.siemens.

# Bottom Connector (Lumberg)

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3LUC

## Fault Description

### Fault Symptoms for customers:

Charging problems.

Problems with external loudspeaker or microphone when using a car kit.

Problems with accessories connected at the bottom connector.

Problems with SW booting.

### Fault Symptom on GSM-Tester:

- This problem cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the bottom connector. Watch for dry joints!

#### Repair by component change

Use hot air blower remove defective bottom connector.

Avoid excessive heat!

Watch surrounding components!

- Resolder new bottom connector afterwards.

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

- Bottom Connector SL45/6688

Part-Number:	 L36334-Z93-C262

#### Jigs and Tools

- Hot Air Blower

Soldering Iron

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 Board Bottom Connector Side

Figure 2: SL45/6688 Bottom Connector Placement (Top View)

Pin 12

Pin 1

Table 1: SL45/6688 Bottom Connector Pin Description

| Pin | Name | IN/OUT | Notes |
| --- | --- | --- | --- |
| 1 | GND |  |  |
| 2 | HS_LEFT | I/O | Output left channel for stereo-headset. |
| 3 | POWER | I | Charging Current |
| 4 | FBatt+ | O | Power supply for the accessories. |
| 5 | TX | O | Serial interface |
| 6 | RX | I | Serial interface |
| 7 | ZUB_CLK | I/O | Clock line for accessory bus Use as DTC In data operation |
| 8 | ZUB_DATA | I/O | Data line for accessory bus. Use as CTS in data operation |
| 9 | GND_MIC |  | For external microphone |
| 10 | MICP2 | I | External microphone |
| 11 | HS_RIGHT | O | Output right channel for stereo-headset |
| 12 | LGND |  | For external loudspeaker |

# Display Connector

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3DIC

## Fault Description

### Fault Symptoms for customers:

Display problems, like missing lines or columns on the LCD or display contrast problems.

### Fault Symptom on GSM-Tester:

- Display test fails.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the status of the display connector. Watch for oxidation and dry solder joints.

Mechanically check the opening / closing mechanism.

#### Repair by component change

- Use hot air to remove defective connector

Avoid excessive heat!

Watch surrounding components!!

- Resolder new connector afterwards

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components	Display connector SL45/6688:

Part-Number:	L36334-Z97-C130

#### Jigs and Tools

- Soldering Iron

Hot Air Blower

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board display connector side

Figure 2: SL45/6688 display connector placement (Top View)

# Keyboard LEDs

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3LED

## Fault Description

### Fault Symptoms for customers:

Keyboard Illumination does not work.

### Fault Symptom on GSM-Tester:

- This fault cannot be detected with a GSM-Tester

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Use the diode test function of a multimeter to check the status of the diode.
The typical voltage drop on the diode is 1.7V when testing the diode function with the multimeter.

#### Repair by component change

- Use soldering iron to remove defective diode

Avoid excessive heat!

Watch surrounding components!

- Resolder new diode afterwards.

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

- LED keyboard SL45/6688

Part-Number:	 L36840-L2057-D670

#### Jigs and Tools

- Hot Air Blower

Soldering Iron

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board keyboard LED Side

Figure 2: SL45/6688 keyboard LED placement (top view)

# Keyboard Connector

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3KCO

## Fault Description

### Fault Symptoms for customers:

Keyboard does not work.

Keyboard Illumination does not work

Mobile does not switch on via keyboard.

### Fault Symptom on GSM-Tester:

The keyboard test fails on the defective button

The illumination fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

#### Repair by component change

- Use soldering iron to remove defective connector

Avoid excessive heat!

Watch surrounding components!

- Resolder new diode afterwards.

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair.

### List of needed material

#### Components

- Keyboard connector SL45/6688

Part-Number:	L36195-Z26-C624

#### Jigs and Tools

- Hot Air Blower

Soldering Iron

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board display connector Side

Figure 2: SL45/6688 board display LED placement

# Irda Diode

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3INF

## Fault Description

### Fault Symptoms for customers:

No infrared connection possible.

### Fault Symptom on GSM-Tester:

- This fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Visually check the status of the IrDa module. Watch for dry solder joints.

Use a reference infrared port (eg. from a notebook) to check the IrDa function. If the notebook recognizes the S35, the infrared function is ok.

Check the status (reverse blocking) of the diode V201 with a multimeter.

#### Repair by component change

Use hot air to remove defective infrared module.

Avoid excessive heat!

Watch surrounding components!!

- Resolder new module afterwards

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair

### List of needed material

#### Components	Infrared diode

Part-Number:	L36810-U6030-D670

#### Jigs and Tools

- Soldering Iron

Hot Air Blower

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board IRDA side

Figure 2: SL45/6688 IRDA placement (Top View)

# Antenna Connector

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3ANC

## Fault Description

### Fault Symptoms for customers:

Network Search when using the external antenna (carkit)

No location update possible on external antenna (carkit)

### Fault Symptom on GSM-Tester:

Output power problems on the external antenna.

No location update possible

### Component Information

The Antenna Connector is a mechanical switch operated by the RF plug of a carkit or, for testing purposes, of an RF clip.

Normally the RF signal goes to and comes from the internal antenna. Whenever an RF plug is plugged into the antenna connector the connection to the internal antenna is opened and the connection to the external antenna socket is made. See drawing below.

From Power Amplifier/

To Receiver

To / From Internal Antenna

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### Diagnosis

Check the output power of the handset with the LSO testprogram. Especially watch the external antenna power!

#### Repair by component change

Use soldering iron to remove defective capacitor.

Avoid excessive heat!

Watch surrounding components!!

- Resolder new capacitor afterwards

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair

### List of needed material

#### Components

- SL45/6688 antenna connector

- Attention: This is not the same connector as C25/S25 !!!

- Part-Number:	L36334-Z93-C272

#### Jigs and Tools

- Soldering Iron

Hot Air Blower

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board antenna connector side

Figure 2: SL45/6688 antenna connector placement (Top View)

# Resistor R384

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

### Fault Code for LSO reporting:	3DRE

## Fault Description

### Fault Symptoms for customers:

- Irregular display illumination

### Fault Symptom on GSM-Tester:

- This fault cannot be detected with a GSM-Tester.

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### General Information

The old display type is replaced by a new one. The replacement has been done in the production in mid-February. The new display module has 4 additional resistors inside.

Also the series resistor R384 located on the mainboard has changed its value from 39 to 0.1.

If the display needs to be exchanged the hardware status of the mainboard and the display module must be checked.

- For boards with a new HW status the resistor R384 has a value of 0.1

- For boards with an old HW status the resistor R384 has a value of 39

#### Diagnosis

Check the status of the resistor with a multimeter.

#### Repair by component change

Use soldering iron to remove defective capacitor.

Avoid excessive heat!

Watch surrounding components!!

#### Repair by SW-Booting

- Not possible!

#### Test

Retest handset after repair

### List of needed material

#### Components	SL45/6688 Resistor R384

Part-Number:	L36245-F4000-F100

#### Jigs and Tools

- Soldering Iron

Hot Air Blower

#### Special Tools

- None

#### Working materials

- Desolder Wick / Braid

Solder

### Drawings

Figure 1: SL45/6688 board R384 side

Figure 2: SL45/6688 R384 placement (Top View)

# Voltage Protection Diode V200

## Affected Units

### Type:					SL45/6688

### Affected IMEIs / Date Codes:	All / All

### Affected SW-Versions:		All

## Fault Description

### Fault Symptoms for customers:

External equipment which needs power via the I/O connector does not work.

### Fault Symptom on GSM-Tester:

BATT+ rises too fast

## Priority:

	........	Mandatory

	........	Repair

	........	Optional

	........	Not Yet Defined

## Repair Documentation

### Description of procedure:

#### General Information

The purpose of the diode V200 is to prevent the feedback of external connected equipment. An external voltage, connected to the I/O connector, may damage the accumulator.

#### Diagnosis

Check the status (reverse blocking) of the diode V200 with a multimeter.

#### Repair by component change

Use soldering iron to remove defective diode.

Avoid excessive heat!

Watch surrounding components!!

Resolder new diode afterwards

#### Repair by SW-Booting

Not possible!

#### Test

Retest handset after repair

### List of needed material

#### Components

#### Diode V200

Part-Number:	L36840-D5035-D670

#### Jigs and Tools

Soldering Iron

Hot Air Blower

#### Special Tools

None

#### Working materials

Desolder Wick / Braid

Solder

#### Drawings

Figure 1: X35 V200 side

Figure 2: X35 V279 placement (Top View)

V200